| OVERALL INSPECTION RESULT -------- PASSED PENDING FAILED N/A |  | Product Photo. |
| --- | --- | --- |

This report reflects the findings at the time and place of inspection only. The final decision to accept or reject the lot rests with the client.

Report Details

| Report No. |  | Product Description |  | Client |
| --- | --- | --- | --- | --- |
| P/O Number |  | Item Number |  | Inspection Date |
| Supplier |  | Factory Name |  | Inspection Location |
| P.O. Quantity |  | Quantity Ready for Inspection (pcs) |  | Inspection Type Pre-Shipment Inspection (PSI) |
| Shipment (ETD) Date |  | Report Date |  | Man-days Spent |
| Reference Sample Available -------- |  | Factory Contact / Attendee |  |  |

Inspection Result Summary

|  | Criteria | Result | Note |
| --- | --- | --- | --- |
| A | Quantity | -------- |  |
| B | Package & Packaging | -------- |  |
| C | Product Style / Construction | -------- |  |
| D | Product(s) Logo, Labelling & Markings | -------- |  |
| E | Accessories | -------- |  |
| F | Measurement Data | -------- |  |
| G | Functional & Performance Tests | -------- |  |
| H | Other Specifications / Requirements | -------- |  |
| I | Product(s) Workmanship | Within AQL |  |

Additional Remarks

| Inspector(s) Date |  | Reviewer / Approved by Date |
| --- | --- | --- |

Sampling Plan · ANSI/ASQ Z1.4

| Visual & Workmanship — SAMPLE SIZE General Level II AQL; Acc.; Rej.; Found. Cr.; 0; —; —. Maj.; 2.5; —; —. Min.; 4.0; —; —. |  | Functional Tests — SAMPLE SIZE Special Level 2 AQL; Acc.; Rej.; Found. Cr.; 0; —; —. Maj.; 2.5; —; —. Min.; 4.0; —; —. |  | Special Checks — SAMPLE SIZE Special Level 1 AQL; Acc.; Rej.; Found. Cr.; 0; —; —. Maj.; 2.5; —; —. Min.; 4.0; —; —. |
| --- | --- | --- | --- | --- |

Quantity

|  |  |  |  | Booking Qty (pcs/sets) |  | Actual Qty (pcs/sets) |  |  | Checked Qty (pcs/sets) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part Number | Color / Finish | Spec / Rev. | P.O. Number | Qty | Carton | Qty | % | Carton | Qty | Carton |
| TOTAL |  |  |  |  |  |  |  |  |  |  |

| Carton numbers selected for inspection |  |
| --- | --- |

Booking Qty = quantity ordered per P.O.   ·   Actual Qty = quantity produced and packed at the time of inspection   ·   Checked Qty = quantity opened and inspected.

| Total cartons in shipment |  | Cartons packed & available |  | Cartons opened for inspection |
| --- | --- | --- | --- | --- |

Package & Packaging

| 2.1 | PACKING VISUAL CHECK | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Packing method and conditions - Retail package artworks - Correct content within the package - Quantity of items & accessories within the package - Presence of correct version of printed literature within the package | RESULT -------- |

| 2.2 | SHIPPING MARKS | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Conform with client specifications - Shipping marks / labels and their position | RESULT -------- |

| 2.3 | BARCODE VISUAL CHECK | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - All the barcodes need to be clean - Barcode type as requested - Positions as per client requirements | RESULT -------- |

| 2.4 | PACKAGING PROTECTION | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Check the individual packaging (poly bag / wrap / box) protects the product from scratches, marks and transit damage. | RESULT -------- |

| 2.5 | CARTON STRENGTH & SEALING | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Verify carton board strength (burst/edge crush) suitable for part weight, proper taping/sealing pattern | RESULT -------- |

| Carton Status | Shipping Carton | L= cm | W= cm | H=cm | G.W.= kg |
| --- | --- | --- | --- | --- | --- |
|  | Inner / Retail Box | L= cm | W= cm | H=cm | G.W.= kg |
| Packaging Method |  |  |  | Carton plies | --- |

Remarks

| Product Storage |  | Master Carton View |
| --- | --- | --- |

| Packing Sealing Method |  | Packing Quality (5ply / 7ply) |
| --- | --- | --- |

| Inner Packing View |  | Product Unit Packing View |
| --- | --- | --- |

| Shipping Marks on Master Carton Front |  | Shipping Marks on Master Carton Side |
| --- | --- | --- |

| Shipping Marks on Inner Box Front |  | Shipping Marks on Inner Box Side |
| --- | --- | --- |

| Barcode on the Shipping Carton |  | Barcode on the Blister Box |
| --- | --- | --- |

| Master Carton Length |  | Master Carton Width |
| --- | --- | --- |

| Master Carton Height |  | Master Carton Weight |
| --- | --- | --- |

| Inner Carton Length |  | Inner Carton Width |
| --- | --- | --- |

| Inner Carton Height |  | Inner Carton Weight |
| --- | --- | --- |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Product Style / Construction

| 3.1 | CHECK THE CONFORMITY OF THE PRODUCT COMPARED TO THE APPROVED SAMPLE | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Same style/materials with approved sample. | RESULT -------- |

| 3.2 | CHECK THE CONFORMITY OF THE PRODUCT COMPARED TO THE TECHNICAL FILE OF THE CLIENT | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Same style/materials with technical file of the client. | RESULT -------- |

| 3.3 | MATERIAL VERIFICATION | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Confirm materials match the approved specification / sample (visual and hand check; supplier CoA or certificate where applicable). | RESULT -------- |

| 3.4 | FABRIC WEIGHT & COMPOSITION | INSP. LEVEL ▸ Special Level 2 |
| --- | --- | --- |
|  | - Verify fabric GSM against the spec with a GSM cutter and scale. - Check the composition claim against the supplier CoA (burn test if in doubt). | RESULT -------- |

| 3.5 | SEAM & STITCH CONSTRUCTION | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Verify stitch type and SPI (stitches per inch) against the tech pack. - Check seam allowance and seam type on main seams. | RESULT -------- |

Remarks

| Product Front View |  | Product Side View |
| --- | --- | --- |

| Product Inner View |  | Product Compared to Approved Sample |
| --- | --- | --- |

| Product Compared to Approved Sample |  | Product Compared to Approved Sample |
| --- | --- | --- |

| Other Product View |  | Other Product View |
| --- | --- | --- |

| Other Product View |  | Other Product View |
| --- | --- | --- |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Product(s) Logo, Labelling & Markings

| 4.1 | LOGO | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Logo quality, colors, dimensions, positioning, and alignment must conform with client specifications. | RESULT -------- |

| 4.2 | LABEL & MARKING TEST | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Correct content within the label & marking. | RESULT -------- |

| 4.3 | BARCODE VISUAL CHECK ON PRODUCT | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - All the barcodes need to be clean. - Barcode type as requested. - Positions as per client requirements. | RESULT -------- |

| 4.4 | WARNING/SAFETY LABELS | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Confirm the safety/warning labels required for the product and destination market are present, correct and legible. | RESULT -------- |

| 4.5 | CARE LABEL CONTENT | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Verify wash/dry/iron/bleach symbols and instructions (ISO 3758 / ASTM care rules) match the fabric and the destination market. | RESULT -------- |

| 4.6 | FIBER CONTENT & ORIGIN LABELS | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Check fiber composition and country-of-origin labels are present, accurate and compliant for the destination market. | RESULT -------- |

Remarks

| Logo |  | Main Label |
| --- | --- | --- |

| Care Label Front View (If applicable) |  | Care Label Back View (If applicable) |
| --- | --- | --- |

| Hang Tag Front View (If applicable) |  | Hang Tag Back View (If applicable) |
| --- | --- | --- |

| Barcode View |  |  |
| --- | --- | --- |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Accessories

| 5.1 | ACCESSORIES CHECK | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Accessories (if any) - Instruction Literature (if any) | RESULT -------- |

Remarks

| Golden sample on the left for Accessories |  | Accessories |
| --- | --- | --- |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Measurement Data

| Insp. Level General Level II |  | Normal Dim's Acc. / Rej. 0 / 1 |  | Sample Size |  | Result -------- |
| --- | --- | --- | --- | --- | --- | --- |

| # | Feature | Spec. | + Tol | - Tol | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Chest width (cm) |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Body length (cm) |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |

Garments: attach size-chart measurement tables per size / colour (POM rows × sizes).

Remarks

| Measurement |  | Measurement |
| --- | --- | --- |

| Measurement |  | Measurement |
| --- | --- | --- |

| Measurement |  | Measurement |
| --- | --- | --- |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |

Functional & Performance Tests

Remove any checkpoint that does not apply to your product.

| 7.1 | WEIGHT CHECK | INSP. LEVEL ▸ 5 unit |
| --- | --- | --- |
|  | - Weight shall match the product specification / approved sample. - Unless otherwise specified, tolerance ±5 %. | RESULT -------- |

| Weight Test |  | Weight Test |
| --- | --- | --- |

| 7.2 | SHIPPING CARTON STACKING TEST | INSP. LEVEL ▸ __ cartons |
| --- | --- | --- |
|  | - One column of export carton will be piled up to a height of 2.5m or as intended to be palletized for four hours. After the test, the bottom carton must show no compression that would affect the carton's shape and no damage to the product inside. | RESULT -------- |

| Shipping Carton Stacking Test |  | Shipping Carton Stacking Test |
| --- | --- | --- |

| 7.3 | SHIPPING CARTON DROP TEST | INSP. LEVEL ▸ 1 carton |
| --- | --- | --- |
|  | - Height and drop sequence according to ISTA 1A procedure. Freefall on hard floor on one corner, three (3) edges, six (6) faces from ..... cm. | RESULT -------- |

| Shipping Carton Drop Test |  | Shipping Carton Drop Test |
| --- | --- | --- |

| 7.4 | FUNCTIONAL TEST | INSP. LEVEL ▸ Special Level 3 |
| --- | --- | --- |
|  | - Operate each function as intended for use - The product needs to function in compliance with the information provided. | RESULT -------- |

| Functional Test |  | Functional Test |
| --- | --- | --- |

| 7.5 | PULL TEST | INSP. LEVEL ▸ Special Level 3 |
| --- | --- | --- |
|  | - Pull test on small parts - Use a pull gauge, 80N, to pull on multiple accessories on the same piece. - If no tool available during Inspection, check by hand with appropriate force. | RESULT -------- |

| Pull Test |  | Pull Test |
| --- | --- | --- |

| 7.6 | ADHESIVE TEST | INSP. LEVEL ▸ Special Level 1 |
| --- | --- | --- |
|  | - Apply 3M-type tape over the printing/screening and pull off at a 45° angle. - Markings must remain clearly legible; adhesive labels must not work loose or curl at the edges. | RESULT -------- |

| Adhesion Test |  | Adhesion Test |
| --- | --- | --- |

| 7.7 | RUB TEST | INSP. LEVEL ▸ Special Level 3 |
| --- | --- | --- |
|  | - Rub the item 10 times over a 10 cm distance with a force of 9N (1kg). - There should be no color transfer greater than 4 on a grey scale (between the original white cloth and the colored one) after the dry test. - There should be no color transfer greater than 3-4 on a grey scale (between the original white cloth and the colored one) after the wet test. | RESULT -------- |

| Rub Test |  | Rub Test |
| --- | --- | --- |

| 7.8 | SMELL CHECK | INSP. LEVEL ▸ Special Level 3 |
| --- | --- | --- |
|  | - Check the smell of the product. It should not have any solvent, mould or strong odor. | RESULT -------- |

| Smell Test |  | Smell Test |
| --- | --- | --- |

| 7.9 | BARCODE SCAN TEST | INSP. LEVEL ▸ Special Level 1 |
| --- | --- | --- |
|  | - All the barcodes need to be clean and readable with a barcode reader and the reading must correspond to client's information and printed numbers / characters. - Barcode type as requested. - Positions as per client requirements | RESULT -------- |

| 7.10 | SEAM STRENGTH TEST | INSP. LEVEL ▸ Special Level 1 |
| --- | --- | --- |
|  | - Apply firm hand pressure across main seams (or a seam-strength gauge per spec). - No seam opening, slippage or stitch breakage allowed. | RESULT -------- |

| 7.11 | SIZE & FIT CHECK | INSP. LEVEL ▸ Special Level 1 |
| --- | --- | --- |
|  | - Lay-measure or try on against the size chart, per size in the size set. - Fit, closure alignment and proportions match the approved sample. | RESULT -------- |

| 7.12 | NEEDLE / METAL DETECTION | INSP. LEVEL ▸ Special Level 1 |
| --- | --- | --- |
|  | - Pass finished units through a calibrated needle detector. - No metal contamination allowed; record the detector calibration check. | RESULT -------- |

| 7.13 | ZIPPER / CLOSURE CYCLE TEST | INSP. LEVEL ▸ Special Level 1 |
| --- | --- | --- |
|  | - Operate zippers, buttons and snaps through 30 open-close cycles. - No breakage, detachment or malfunction allowed. | RESULT -------- |

| 7.14 | COLOR / SHADE CHECK (LIGHT BOX) | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Compare color/shading in a standard light box against the color standard or approved sample; keep fabric direction consistent (it affects luster). - Minimum grey scale 4 — both against the approved sample and within one product. | RESULT -------- |

| Barcode Scan Test |  | Barcode Scan Test |
| --- | --- | --- |

| [Photo Description] |  | [Photo Description] |
| --- | --- | --- |

Remarks

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Other (Client) Specifications / Requirements

| 8.1 | CLIENT REQUIREMENT | INSP. LEVEL ▸ — |
| --- | --- | --- |
|  |  | RESULT -------- |

Remarks

| Other (Client) Specifications |  | Other (Client) Specifications |
| --- | --- | --- |

| Other (Client) Specifications |  | Other (Client) Specifications |
| --- | --- | --- |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Product(s) Workmanship

| 9.1 | PRODUCT VISUAL CHECK | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Product physical conditions, appearance & cleanliness | RESULT Within AQL |

| 9.2 | SURFACE DEFECTS | INSP. LEVEL ▸ General Level II |
| --- | --- | --- |
|  | - Inspect for stains, holes, snags, loose threads, needle damage and shade variation across visible surfaces. | RESULT Within AQL |

Defects Found

Critical — may harm the user or breach mandatory regulations (Acc = 0).   Major — reduces usability or salability; likely to cause customer returns.   Minor — deviation unlikely to reduce usability or salability.

| Description | Critical | Major | Minor |
| --- | --- | --- | --- |
| Total found defect |  |  |  |
| Number of defects allowed |  |  |  |
| Checked pieces |  |  |  |

Defect Photos

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

| [Defect Description] |  | [Defect Description] |
| --- | --- | --- |
| *Remove this block if no defects found |  | *Remove this block if no defects found |

Inspection Conditions & Site Observations

| Working Environment |  |
| --- | --- |
| Inspection area and lighting adequate for reliable judgement | -------- |
| Housekeeping and cleanliness of the inspection area | -------- |
| Overall state of the production facility | -------- |
| Inspection Integrity |  |
| Factory staff support for sample handling and repacking | -------- |
| Measuring instruments provided with valid calibration records | -------- |
| Cooperation of factory management during the visit | -------- |
| Samples selected at random by the inspector from sealed export cartons | -------- |
| Restrictions imposed on the inspector (access, time, photography) | -------- |
| Inspection completed in full within the scheduled time | -------- |

| Factory Entrance / Signboard |  | Production Facility Overview |
| --- | --- | --- |

| Inspection Area & Lighting |  | Housekeeping of Inspection Area |
| --- | --- | --- |

Reference Photo

K1 – Signed inspection summary acknowledged by factory representative

— End of report  ·  Report format provided by InspectionService.com —